### ספר הפרויקט

יוגש באתר הקורס ב MOODLE (לאחר אישור המנחה) עד לתאריך שפורסם.

הגשת ספר הפרויקט בזמן – סטודנטים שלא יגישו בזמן ולפי ההנחיות, לא יוכלו להציג את הפרויקט במועד שנקבע ויצטרכו לתאם מועד חדש.

ספר הפרויקט מסכם את עבודת הסטודנטים בפרויקט.

ספר הפרויקט יכלול את הפרקים הבאים:

  - תקציר (עמוד אחד)

  - הקדמה ורקע תיאורטי

  - תכנון ו\או סימולציה

  - מימוש

  - תוצאות ובדיקות (עם דגש לדרישות הכמותיות)

  - סיכום, מסקנות והצעות להמשך

  - תיעוד הפרויקט

  - רשימת מקורות (כמקובל בספרות מדעית)

יש להיוועץ עם המנחה בפרויקט, על מנת להבין, איזה פרקים יש לכלול בספר הפרויקט בהתאם לאופיו של הפרויקט שלכם.

יש להגיש את הספר, לפי התבנית המצורפת.

ניתן להגיש את הספר באנגלית, תוך הקפדה על התבנית. גודל הקובץ הוא לכל היותר 10  מגה.

סדר הגודל המומלץ לספר הפרויקט הוא 20 עמודים.

ספר הפרויקט לא יכלול את קטעי הקוד ,דפי נתונים וכו'. במידה ויש התייחסות ספציפית בספר הפרויקט לחלק מהקוד או דפי הנתונים וכו', ניתן לצרף את החומר הרלוונטי בלבד וכנספח לספר הפרויקט.

SyncRun

| 25-1-1-3408 | Project Number: |
| --- | --- |

Project Report

| Student: | Matan Granot | ID: | 211696406 |
| --- | --- | --- | --- |
| Student: | Yogev Grinblatt | ID: | 212365787 |

| Supervisor: | Dr. Lior Arbel |
| --- | --- |

| Project Carried Out at: | Tel Aviv University |
| --- | --- |

Contents

Abstract………..	2

1	Introduction	3

2	Theoretical background	4

3	Simulation..	5

4	Implementation	6

4.1  Hardware Description	6

4.2  Software Description	7

5	Analysis of results	8

6	Conclusions and further work	9

7	Project Documentation	10

8	References..	11

Appendix  A.  Subject 1	12

Appendix  B   Subject 2	13

List of figures

Figure 1: Block diagram	2

Figure 2: The System	6

Figure 3: Amplifier Input Circuit	6

List of tables

Table 1: Results	8

List of equations

# Abstract

The abstract summarizes the essence of the work (project topic, project purpose, and project deliverable). Further to the description, a block diagram of the project should be presented .

Figure 1: Block diagram

The chapters, illustrations and tables should be numbered automatically, as done here. The Table of Contents should also be generated.

Figures and illustrations can be generated either from a POWERPOINT slide (as done here) or added (JPG, TIFF, BMP) as done in Chapter 4, or from a PDF document as done in Subsection 4.1.

An example of the table can be seen in subchapter 5.1.

# Introduction

Background and Motivation

The integration of music into athletic training has become increasingly popular, as rhythm and tempo can significantly enhance a runner's motivation, improve physical coordination, and provide a smoother running experience. However, traditional music playback applications typically offer a fixed tempo or rely on manual speed adjustments, which fail to adapt to the fluctuating pace of a runner throughout a specific workout. In reality, the natural beats per minute (BPM) of most standard songs are slower than an average runner's cadence. This limitation creates a gap at the intersection of musical digital signal processing (DSP) and sports applications, presenting an opportunity for a system capable of analyzing a user's planned or recorded physical rhythm and seamlessly adjusting the audio stream beforehand to match it.

Project Objectives

The core objective of this project, "SyncRun – Align Music to Run" (Project 3408), is to design and implement a software system capable of adjusting the playback rate of a rhythmic music track to match the offline physiological metrics (step rhythm) of a runner. Using a fully pre-determined runner step signal, the system processes the audio prior to playback, calculating an optimal speed-up or speed-down ratio for each specific segment of the run one time beforehand. The goal is to synchronize the timing of the music's beats with the runner's steps using time-stretch algorithms, improving flow and motivation without altering the song's original pitch or introducing unnatural audio distortions.

[Insert a high-level Block Diagram here illustrating the offline system architecture, from pre-determined step tracking and audio pre-processing to the modified audio compilation]

The Approach to Solving the Problem

To solve the offline synchronization problem, the system employs a multi-step digital signal processing pipeline implemented in Python. The approach is broken down into the following stages:

- Sequence Extraction: The librosa library is used to analyze the song offline, determining the beat timestamps and overall tempo (BPM).

- Alignment and Ratio Optimization: The system generates two time-stamped vectors—one for the song's beats and one for the pre-determined runner steps. The track is divided into discrete 2.5-second segments. For each segment, the algorithm computes the optimal playback speed ratio by minimizing the L1 distance between the runner's step timestamps and the scaled song's beat timestamps.

- Harmonic-Percussive Separation (HPS): Before time-scaling, the system applies a Short-Time Fourier Transform (STFT) and median filters to separate the original audio into its sustained, pitched sounds (harmonic content) and sharp, transient drum hits (percussive content).

- Time-Scale Modification (TSM): Custom-built TSM algorithms stretch or compress the separated audio segments to the calculated ratios without shifting the pitch, after which they are merged to output the final, modified song.

[Insert STFT Spectrogram plots here showing a song snippet before and after the Harmonic-Percussive Separation process]

Comparison Against Existing Work and Algorithms

This project differentiates itself from both commercial fitness applications and existing TSM libraries:

  - Application-Level Comparison: Conventional fitness music applications (such as Spotify Running) generally rely on pre-set tempo playlists or basic manual speed adjustments, which offer limited adaptability. Other alternatives utilize metronome-based synchronization, which provides fixed rhythmic cues but lacks the natural, motivating experience of music. In contrast, this project offers precise, mathematical beat-to-step alignment mapped directly to a runner's specific workout routine.

  - Algorithmic-Level Comparison: Rather than treating audio manipulation as a black box by exclusively using off-the-shelf TSM libraries (e.g., Rubber Band), this project focuses on a deep, fundamental DSP implementation. The WSOLA (Waveform Similarity Overlap-Add) and Phase Vocoder algorithms are implemented from scratch in Python. Grounded in the theoretical framework of Driedger and Müller's A Review of Time-Scale Modification of Music Signals, applying these custom TSM techniques independently to separated harmonic and percussive components allows for a higher quality manipulation that aims to reduce the audio artifacts commonly found in standard, unseparated time-stretching methods.

Scope and Evaluation

Because the system processes the entire track and step signal offline, the heavy computational load is handled beforehand, ensuring a seamless, uninterrupted playback experience. The custom TSM algorithms must operate stably across speed-up ratios ranging from 0.8× to 1.5×. To ensure a high-quality result, the system's output is evaluated against strict quantitative criteria, including average beat alignment error, segment latency, and the total elapsed time required for the offline processing phase.

[Insert a table here summarizing the quantitative performance metrics and criteria results (e.g., Average Beat Alignment Error: ~15.67 ms, Total Elapsed Processing Time: ~38.47 sec)]

# Theoretical background

[Insert a high-level theoretical pipeline diagram here summarizing the flow: Ratio Optimization → STFT → Harmonic-Percussive Separation → Time-Scale Modification]

Alignment and Ratio Optimization

The foundational step in synchronizing the audio offline involves computing an optimal speed-up or speed-down ratio, denoted as r, for discrete segments of the audio. The algorithm calculates this scalar by minimizing the absolute difference between the scaled song timestamps and the pre-determined runner step timestamps.

- Mathematical Formulation: To achieve robust alignment, the optimization is framed as minimizing the L1 norm between the two temporal vectors, expressed mathematically as:

minri|r⋅xi-yi|

where xi represents the elements of the song timestamp vector and yi represents the target runner step timestamps.

  - Weighted Median Solution: This objective function constitutes a Least Absolute Deviations (LAD) linear regression forced through the origin. Based on the theoretical framework of Bloomfield and Steiger (1984), the global minimum for the scaling factor r is computed exactly by finding the weighted median of the component ratios yixi, where the weights are the magnitudes |xi|. This L1 optimization was selected because it provides an exact, non-iterative alignment solution that is highly resistant to outliers, such as irregular runner steps or false-positive beat detections.

Short-Time Fourier Transform (STFT)

To analyze and manipulate the frequency content of the audio over time, the system transforms the time-domain audio signal into a time-frequency representation using the Short-Time Fourier Transform (STFT). This transformation serves as the fundamental basis for both the subsequent separation and frequency-domain stretching stages.

- Core Concept: The STFT works by sliding a short-duration analysis window over the audio signal and calculating the discrete Fourier transform for each overlapping segment.

- Mathematical Definition: The STFT is calculated as follows:

Xm,k=n=0N-1xn+mH⋅wn⋅e-j2πNkn

where x is the input signal, w is the analysis window function of length N, H is the hop size (the number of samples between successive windows), m is the frame index, and k is the frequency bin index. The output is a complex-valued spectrogram that holds both the magnitude and phase information necessary for advanced audio processing.

Harmonic-Percussive Separation (HPS)

Applying uniform time-stretching to a dense, mixed audio track often introduces distinct artifacts, as harmonic and percussive sounds react differently to processing. To mitigate this and preserve audio quality, the system separates the signal into two distinct components prior to modification.

- Spectrogram Representation: Using the magnitude of the STFT spectrogram, harmonic sounds (like vocals or sustained chords) appear as horizontal lines representing sustained frequencies over time, whereas percussive sounds (like drum hits) appear as vertical lines representing broadband energy impulses over a short duration.

- Median Filtering: The separation is achieved by applying orthogonal median filters across the STFT magnitude spectrogram. A horizontal median filter isolates and extracts the sustained harmonic content, while a vertical median filter captures the sharp, transient percussive hits.

- Mathematical Definition: The Harmonic-Percussive Source Separation (HPSS) masks are calculated as follows:

Mharm=HsmoothpowerHsmoothpower+margin⋅Psmoothpower+ϵ

Mperc=PsmoothpowerPsmoothpower+margin⋅Hsmoothpower+ϵ

where Hsmooth is the magnitude spectrogram smoothed across the time axis, Psmooth is the magnitude spectrogram smoothed across the frequency axis, power is the exponent used to scale the magnitudes, margin is the multiplier used to enforce strict separation between the tracks, and ϵ is a very small constant added to the denominator to prevent division by zero.

[Insert STFT Spectrogram plots here demonstrating the visual distinction between the horizontal (Harmonic) and vertical (Percussive) components]

Time-Scale Modification (TSM)

Time-Scale Modification (TSM) refers to a class of digital signal processing algorithms designed to alter the playback duration of an audio signal without affecting its perceived pitch. Following established theoretical frameworks, our project implements two distinct TSM techniques tailored specifically to the separated audio components:

- WSOLA (Waveform Similarity Overlap-Add): WSOLA is a time-domain TSM algorithm that is highly effective for processing transient, percussive signals. It operates by breaking the signal into overlapping frames and shifting them to compress or stretch the timeline. To prevent audible artifacts such as phase cancellations or clicking, WSOLA uses cross-correlation to search for regions of high waveform similarity. This ensures that overlapping similar time-domain waveforms align seamlessly before they are added together, preserving the punch and timing of drum hits.

- Mathematical Definition: The WSOLA analysis hop and optimal shift are calculated as follows:

Ha=Hs⋅rate

Δopt=argmaxδSearchRegion⋅Template

where Hs is the synthesis hop size (the spacing in the output audio), rate is the desired time-stretch ratio, Ha is the resulting analysis hop size (the spacing in the input audio), Template is the overlapping section of the already-constructed output signal, SearchRegion is the constrained window of input audio being scanned, δ is the currently tested shift distance, and Δopt is the optimal time shift that produces the maximum cross-correlation to perfectly align the overlapping waveforms.

- Phase Vocoder (PV-TSM): The Phase Vocoder is a frequency-domain TSM algorithm best suited for harmonic and pitched content. It utilizes the STFT to independently control time and pitch by manipulating the signal's magnitude and adjusting frequency phases. When stretching or compressing the audio frames to match the new ratio r, the phase values of the frequency bins are explicitly updated and unwrapped. This maintains phase coherence across overlapping frames, ensuring that sustained pitches remain natural and free of the "warbling" artifacts typical in basic time-stretching.

- Mathematical Definition: The Phase Vocoder phase accumulation is calculated as follows:

Δϕexpectedk=2π⋅k⋅HNfft

ϕoutt,k=ϕoutt-1,k+Δϕexpectedk+Δϕwrappedk

where k is the frequency bin index, H is the hop size, Nfft is the FFT window size, Δϕexpectedk is the theoretically predicted phase advance for a steady frequency, Δϕwrappedk is the actual measured phase difference wrapped to the -π,π boundary, and ϕoutt,k is the final accumulated phase applied to the new complex spectrogram to maintain the original pitch.

[Insert WSOLA and Phase Vocoder figures]

# Simulation

Simulation Environment and Setup

The simulation environment for the "SyncRun" project was developed entirely in Python. Because the project focuses on an offline, pre-processing approach, the simulation does not require real-time hardware tracking but instead relies on computational digital signal processing (DSP) libraries. To conduct controlled experimentation and validate the algorithms, the system utilizes synthetic runner signals to simulate various step sequences and workout paces.

The primary audio inputs consisted of real, simple rhythmic songs (e.g., "Seven Nation Army"), which were processed alongside the synthetic step vectors. Sequence extraction, including the detection of song onsets and global tempo (BPM), was implemented using the librosa library.

Execution Pipeline and Methodology

To test the full scope of the system, the simulation executed the entire offline processing pipeline from end to end:

- Vector Generation: Two primary time-stamped vectors were generated: one containing the simulated runner steps and the other containing the song's extracted beat timestamps.

- Segmented Alignment: The simulation divided the track into 2.5-second segments. For each segment, the algorithm computed the optimal speed-up ratio by minimizing the L1 distance between the step and beat vectors.

- Stability Testing: The simulation rigorously tested the Time-Scale Modification (TSM) module to ensure it operated stably across a wide range of speed-up and speed-down ratios, specifically targeting values between 0.8× and 1.5×.

- Audio Synthesis: The system separated the audio using Harmonic-Percussive Separation (HPS) and applied the custom WSOLA and Phase Vocoder algorithms respectively, before merging the tracks back together into a final output file.

[Insert a visual plot here showing the overlay of the synthetic runner step signal against the scaled song beat vector for a sample segment]

Performance Metrics and Results

To quantify the success of the offline simulation, the system's performance was measured against strict criteria focusing on alignment accuracy and computational efficiency. The tests demonstrated that the algorithmic pipeline successfully maintained tight synchronization without introducing prohibitive processing delays.

The quantitative results of the simulation are summarized below:

| Performance Criteria | Result | Description |
| --- | --- | --- |
| Average Beat Alignment Error | 15.67 ms | The average temporal difference between a runner's step and the corresponding scaled musical beat, confirming highly accurate phase alignment. |
| Average Segment Latency | 475 ms | The average computational time required to process an individual 2.5-second segment of audio. |
| Total Elapsed Time | 38.47 sec | The overall time required to completely process, time-stretch, and synthesize the offline audio track. |

[Insert STFT correlation or latency distribution plots here to visually represent the system's performance stability over the duration of the track]

# Implementation (Software implementation)

The entire computational pipeline for the "SyncRun" project is implemented in Python, utilizing an array of specialized digital signal processing (DSP) and numerical computing libraries. The system relies heavily on NumPy for vectorized array operations, SciPy for signal transforms and filtering, Librosa for musical feature extraction, and Soundfile for high-fidelity audio output. To maintain optimal performance during development and testing, the system caches pre-processed audio arrays and metadata into .npy files, avoiding redundant processing times.

Pre-Processing and Vector Alignment

The initial phase of the implementation focuses on generating the absolute temporal markers for both the audio track and the simulated runner, followed by computing the optimal speed-up ratios for discrete segments of the song.

- Feature Extraction and Synthetic Step Generation

The system processes the input track (e.g., "Seven Nation Army") by utilizing Librosa's beat_track function to extract the global tempo (BPM) and absolute timestamps of the musical beats. Because the project tests the algorithms using simulated runner dynamics, a synthetic step vector is generated based on a nominal speed ratio (r_nom) derived from the target Steps Per Minute (SPM) relative to the song's BPM. To simulate natural human inconsistency, a stochastic variance is introduced into the step timestamps using a normal distribution. A standard deviation parameter of 0.05 is applied relative to the duration between steps, ensuring the algorithm must handle realistic temporal fluctuations rather than a perfectly rigid grid.

- The L1 Weighted Median Optimizer

The global beat and step vectors are divided into overlapping segments of precisely 2.5 seconds. The core of the temporal alignment is executed by the phase_pp_l1_shifted algorithm, which determines the optimal playback ratio (ropt) for each segment.

The algorithm anchors the time-stretch to the segment's starting timestamp (tstart) and accounts for a cumulative temporal drift (d) inherited from previous segments. The optimization is implemented mathematically as follows:

- Relative Alignment: The relative timing of the music beats within the current segment is isolated.

- Target Adjustment: The target run steps are adjusted by stripping both the start time and the inherited delay.

- Weighted Median Calculation: To find the optimal L1-norm fit, the algorithm calculates individual stretch candidates and assigns corresponding weights equivalent to the relative beat timings. The vectors are sorted, and a cumulative sum of the weights is computed. The optimal candidate is found at the index where the cumulative weight surpasses half the total weight.

- Drift Tracking: The chosen ratio (ropt) is returned to the main loop, where the cumulative delay is immediately updated for the next segment using the formula drift[i+1] = drift[i] + segment * (1/r - 1).

Short-Time Fourier Transform (STFT)

Before the distinct Time-Scale Modification algorithms can be applied, the audio is converted into a time-frequency domain matrix.

- STFT Parameters: The compute_stft_custom function utilizes scipy.signal.stft with a window size (nperseg) of 2048 samples and an overlap (noverlap) of 1536 samples.

- Windowing and Hop Size: A standard Hann window ("hann") is applied to reduce spectral leakage, yielding an effective hop length of 512 samples (hop_length = nperseg - noverlap).

Harmonic-Percussive Separation (HPS)

The complex STFT matrix (Zxx) is passed into a custom Harmonic-Percussive Source Separation (HPSS) module designed to isolate pitched instruments from transient drums using orthogonal filters.

- Median Filtering: The algorithm computes the magnitude of the spectrogram and applies a 1D median filter along the temporal axis to smooth the harmonic content (size=(1, 31)) and along the frequency axis to smooth the percussive content (size=(31, 1)).

- Soft Masking: Soft masks are generated to separate the signals without introducing harsh numerical cutoffs. The masks are calculated using a power coefficient of 2.0 and a separation margin of 1.2 to enforce strict isolation between the matrices. For example, the harmonic mask formula is computed as (harm_smooth ** power) / ((harm_smooth ** power) + ((margin * perc_smooth) ** power) + eps).

- Residual Matrix: To ensure no audio information is lost during the split, a residual mask is computed by subtracting the harmonic and percussive masks from 1.0 (1.0 - (harm_mask + perc_mask)), clipped cleanly between 0.0 and 1.0. The original complex STFT is then multiplied by these three masks, resulting in three distinct sub-spectrograms: Harmonic, Percussive, and Residual.

Time-Scale Modification Engines

With the audio correctly isolated, the system routes the signals to two distinct, custom-built Time-Scale Modification engines, each driven by the targeted speed ratio (ropt) for the current 2.5-second chunk.

- Frequency-Domain Stretch: The Phase Vocoder

The custom_phase_vocoder engine processes the Harmonic and Residual spectrogram matrices. Because stretching frequency bins alters their natural oscillation cycles, phase coherence must be manually recalculated to prevent severe warbling artifacts.

- Time Interpolation: The engine calculates a new time grid based on the required stretch rate. For each new output frame, the algorithm finds the nearest original frames (col0 and col1) and linearly interpolates the magnitude values using a fractional alpha weight (alpha = t - col0).

- Phase Unwrapping and Accumulation: A global phase advance vector is computed as phi_advance = 2 * np.pi * k * hop_length / n_fft. For every frequency bin, the phase delta (dp) between the surrounding original frames is extracted, normalized, and wrapped strictly between -π and π using the arithmetic logic dp = dp - 2 * np.pi * np.round(dp / (2 * np.pi)). The true phase is then accumulated into the ongoing current_phase state, which is merged with the interpolated magnitude array to reconstruct the modified complex spectrogram.

- Time-Domain Stretch: Custom WSOLA

The custom_wsola (Waveform Similarity Overlap-Add) engine directly manipulates the Percussive time-domain signal. Because drums contain sharp transients, processing them in the frequency domain causes "smearing"; WSOLA avoids this by keeping the waveform intact and carefully overlapping segments.

- Buffer Initialization: The function receives the current percussive audio alongside a leftover input buffer and output tail from the previous segment loop, concatenating them to ensure gapless processing.

- Cross-Correlation Search: The engine uses a predefined synthesis hop size (syn_hop = 512) and an analysis window size (win_size = 2048). Rather than strictly pasting audio at the target analysis hop point (ana_hop = int(syn_hop * rate)), the algorithm extracts a localized search region constrained by a delta boundary (delta = 512). It utilizes scipy.signal.correlate (in valid mode) to slide a template of the current output against the search region, seeking the position of highest waveform similarity (best_shift = np.argmax(corr) - delta).

- Overlap-Add: The optimally aligned waveform segment is scaled by a Hanning window (w = np.hanning(win_size)) to crossfade the boundaries and is added into the output array.

- 6. Synthesis and Output Generation

Following the independent stretching processes, the Harmonic and Residual matrices are reverted to the time domain using an Inverse STFT (istft) via the reconstruct_signal_custom function. To finalize the chunk, the algorithm trims the arrays to the shortest minimum length (min_len) to prevent dimension-mismatch errors and algebraically sums the separated harmonic, percussive, and residual audio arrays together.

The completed chunks are aggregated sequentially. Once the entire song has been processed and compiled into final_song, the system applies a click track mapping directly to the runner's simulated steps via the add_clicks utility—overlaying exponential decay click envelopes directly onto the output file. The final dynamic synchronized audio is exported via the Soundfile library to a .wav file, providing a complete, seamless representation of the pre-processed runner synchronization pipeline.

# Analysis of results

Comparisons of simulation results to real-time work (as well as simulations for the alternative algorithms presented in the theoretical background section, if real-time realization is not yet available, alternative algorithms must be compared).

Table 1: Results

|  | Parameter | Simulation | Real Time | Alternative Algorithm |
| --- | --- | --- | --- | --- |
|  | Gain | 8 dB | 7.5 dB | 7 dB |
|  | Noise Level | -30 dBm | -50 dBm | -58 dBm |

# Conclusions and further work

This is the most important chapter. This section should include:

- Examining the project results against the goals set in the first place

- Suggestions to improve system performance

- Possibilities for future (development / research) activities

# Project Documentation

All project deliverables are to be documented (e.g. GitHub), this final project report should only include their description:

- Description of the project documentation (Hardware, software, additional simulations, user guide, etc.)

- The Documentation location (e.g. the GitHub link)

- Description of the project files

# References

Here are some examples of references, and how they should be included.

Books:

- A. M. Bronstein, M. M. Bronstein, and R. Kimmel. "Numerical geometry of non-rigid shapes”, Springer-Verlag New York Inc, 2008.

Papers:

- G. B. Giannakis , "Highlights of Signal Processing for Communications",  IEEE Signal Processing Magazine, Vol. 16, no 2, pp. 14-49, March 1999

Datasheet

- “Spartan-3A DSP FPGA Family Data Sheet”, XILINX Product Specification DS610,   October 4, 2010. http://www.xilinx.com/support/documentation/data_sheets/ds610.pdf

Application Note :

- X.  Zhang, "Developing a CCStudio 2.0 DSP/BIOS Application for FLASH Booting on the TMS320C5402 DSK", TI Application Report SPRA661A, November 2000. http://www.ti.com/lit/an/spra661a/spra661a.pdf

User's Guide:

- "TMS320C6201/6701 Evaluation Module User's Guide", SPRU269F, August 2002

- Code Composer Studio IDE 2.0 online help.

Links:

- "Dolby E Multichannel Coding for DTV Audio Production and Distribution", http://www.dolby.com/tech/m.br.9903.epaper.pdf

- G. Welch and G. Bishop – "An Introduction to the Kalman Filter",  http://www.cs.unc.edu/~welch/kalman/kalman_filter/kalman.html#pgfId-11854

- C. Mercer, "Smoothing Spectral Data", The PROSIG Digital Signal Processing Tutorials,   http://www.prosig.com/signal-processing/smoothingspectra.html

Subject 1

If you need to go into details of a specific subject, you should do it here.

A dedicated appendix should be included for each subject.

Subject 2

A single page length at most.

The abstract should allow a person who does not read all chapters of the project book to have a complete picture of the nature of the work.

Notice the insertion of the FOOTNOTE, and the links from this page to the chapters in which the examples appear.